PetroTech Energy Ltd.

Digital Operations Team — Weekly Technical Report

# Week 1 Technical Report

Production & Drilling Performance Review — 01 July 2026

Prepared by: Macdonald, Graduate Digital Petroleum Engineer

# 1. Production Analysis

Daily production data across the Alpha, Bravo, and Charlie fields was queried to identify strong performers and wells requiring intervention. Results are summarized below.

| Well | Field | Status | Oil (bbl/day) |
| --- | --- | --- | --- |
| A101 | Alpha | Active | 3,150 |
| A102 | Alpha | Active | 1,950 |
| B101 | Bravo | Active | 2,810 |
| C101 | Charlie | Shut-in | 0 |
| C102 | Charlie | Active | 3,550 |

## Which field appears strongest?

Charlie posts the single highest producer in the asset — C102 at 3,550 bbl/day — but the field is dragged down by C101, which is currently shut-in and contributing zero barrels. Bravo's single well, B101, is a solid, consistent performer at 2,810 bbl/day. Alpha runs two wells with combined output similar to Bravo alone, but neither individual well matches C102's rate. On a per-active-well basis, Charlie and Alpha are close, with Bravo close behind; the real story is Charlie's shut-in well rather than any field being categorically weak.

## Which wells require attention?

- C101 (Charlie) — shut-in, producing zero barrels; root cause should be confirmed before returning to service.

- A102 (Alpha) — lowest active producer at 1,950 bbl/day; worth a diagnostic check (pump efficiency, drawdown) to see if it is underperforming its potential.

## Highest producers

C102 (3,550 bbl/day) and A101 (3,150 bbl/day) are the top performers and should be monitored closely to sustain current rates.

# 2. Drilling Analysis

Drilling performance was reviewed across three active rigs to identify efficiency issues and non-productive time (NPT) drivers.

| Rig | Well | Depth (m) | ROP (m/hr) | Bit Type | NPT (hr) |
| --- | --- | --- | --- | --- | --- |
| Rig-21 | 6 | 850 | 18.4 | PDC | 2 |
| Rig-19 | 7 | 1,100 | 9.5 | Roller Cone | 7 |
| Rig-25 | 8 | 1,320 | 21.3 | PDC | 1 |
| Rig-21 | 9 | 950 | 13.2 | Hybrid | 4 |
| Rig-19 | 10 | 780 | 8.4 | Roller Cone | 9 |

## Which rigs have drilling issues?

Rig-19 is the clear underperformer, running both of its wells below a 10 m/hr ROP threshold (9.5 and 8.4 m/hr) and logging the highest combined NPT in the dataset (7 and 9 hours respectively). By contrast, Rig-21 and Rig-25 are both delivering ROPs above 13 m/hr with comparatively low NPT.

## What likely caused low ROP?

Both of Rig-19's low-ROP wells were drilled with Roller Cone bits, whereas the faster wells used PDC or Hybrid bits. This points to bit selection as the primary suspect — Roller Cone bits are generally slower in the formations being drilled here — compounded by the elevated NPT, which suggests additional mechanical or tripping delays rather than formation hardness alone.

## Which wells should be investigated first?

Well 10 (Rig-19) should be reviewed first — it combines the lowest ROP (8.4 m/hr) with the highest NPT (9 hours) in the dataset. Well 7 (Rig-19) should follow, as it shares the same rig and bit-type pattern. A bit-selection review for Rig-19, alongside an NPT breakdown to separate mechanical downtime from formation-driven slowdowns, is recommended before its next run.

# Recommended Actions

- Investigate the shut-in status of C101 and develop a return-to-production plan.

- Run a diagnostic on A102 to identify the cause of below-average output.

- Switch Rig-19 to a PDC or Hybrid bit on its next well, matching the higher-performing rigs.

- Conduct an NPT breakdown on Rig-19 to isolate mechanical vs. formation-driven delays.